浙江孚泰科技有限公司 制冷剂产品

R22

R22 is a colorless gas under ordinary temperature and a colorless & transparent liquid under the pressure of itself, Vnon-toxic & inflammable with the excellent thermo-stability and chemical stability, unable to erode metals and it is a  refrigerant for low temperature to ﹣80℃ below. Used in reciprocating compressor, as refrigerant in industrial, commercial and household air-condition system; Also used to produce insecticide and aerosol for spraying paint; To produce V extinguishing agent 1211; As the basic material to produce a wide variety of fluorinated macromolecular compounds.

Properties：

| Index | Unit | R22 |
| --- | --- | --- |
| Chemical formula |  | CHClF2 |
| Molecular weight | g/mol | 86.5 |
| Freezing point | ℃ | -146 |
| Boiling point | ℃ | -40.8 |
| Specific gravity (25℃ liquid) | g/m3 | 1.194 |
| Vapor pressure (25℃) | MPa | 1.04 |
| Critical temperature | ℃ | 96 |
| Critical pressure | MPa | 4.98 |
| Critical density | g/cm3 | 1.221 |
| Vapor heat at boiling point | KJ/kg | 233.5 |
| Water solubility (25℃) | W% | 0.30 |
| Specific heat (25℃ liquid ) | KJ/kg.k | 1.400 |
| ODP |  | 0.05 |
| GWP |  | 0.03 |

Specification：

| Appearance | Colorless and clearness |
| --- | --- |
| Odor | No strange stench |
| Purity, % by weight % ≥ | 99.90 |
| Moisture, ppm by weight% ≤ | 10 |
| Acidity, ppm by weight % ≤ | 1 |
| Evaporation residue, ppm by weight % ≤ | 100 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：13.6kg/22.7kg/950kg net weights  cylinders & ISO tank .

性状用途：

在常温下为无色气体，在自身压力下为无色透明液体，无毒不燃，具有良好的热稳定性和化学稳定性，不腐蚀金属。它是一种低温制冷剂，可得到﹣80℃的制冷温度,用于往复式压缩机，作为工业、商业、家庭空调系统的制冷剂；还可用来做杀虫剂和喷漆的气雾喷射剂，也可用于生产灭火剂1211；是生产各种含氟高分子化合物的基本原料。

物理性质 ：

|  | 单位 | HCFC-22 |
| --- | --- | --- |
| 分子式 |  | CHClF2 |
| 分子量 | g/mol | 86.5 |
| 熔点 | ℃ | -146 |
| 沸点 | ℃ | -40.8 |
| 液体密度(25℃) | 克/厘米3 | 1.194 |
| 蒸气压 (25℃) | MPa | 1.04 |
| 临界温度 | ℃ | 96 |
| 临界压力 | MPa | 4.98 |
| 临界密度 | 克/厘米3 | 1.221 |
| 沸点时蒸发潜热 | 千焦耳/千克 | 233.5 |
| 在水中溶解度 (25℃) | W% | 0.30 |
| 比热 (液体25℃) | 千焦耳/千克度 | 1.257 |
| ODP |  | 0.05 |
| GWP |  | 0.03 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R23

R23 is a high pressure liquefied gas, which was usually used as a refrigerant to substitute CFC-13. And it is also the ideal substitute for Halon with the quality of cleanness, low toxicity, and the excellent effect to fight fire.

Properties：

| Index | Unit | R23 |
| --- | --- | --- |
| Chemical formula |  | CHF3 |
| Molecular weight | g/mol | 70.0 |
| Melting point | ℃ | -152.15 |
| Boiling point | ℃ | -82.2 |
| Specific gravity (25℃ liquid) | g/m3 | 0.669 |
| Vapor pressure (25℃) | MPa | 4.732 |
| Critical temperature | ℃ | 25.95 |
| Critical pressure | KPa | 4.832 |
| Critical density | g/cm3 | 0.524 |
| Vapor heat at boiling point | KJ/kg | 240 |
| Water solubility (25℃) | W% | 0.10 |
| Specific heat (25℃ liquid) | KJ/kg.k | 1.55 |
| ODP |  | 0 |
| GWP |  | 6 |

Specification：

| Index | Quality performance |
| --- | --- |
| Appearance | Colorless , not turbid |
| Odor | No strange stench |
| Purity % ≤ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |

Packing：30kg/40L&380kg/500L,  high pressure steel cylinders.

PS：UN NO.1984, HS Code 29034510.00, CAS 75-46-7, IMDG Page 2184, Class 2.2.

性状用途：

是一种高压液化汽，可用作制冷剂，替代CFC-13，还可用于灭火剂。具有清洁低毒、灭火效果等特点。

物理性质 ：

|  | 单位 | HFC-23 |
| --- | --- | --- |
| 化学式 |  | CHF3 |
| 分子量 | g/mol | 70.0 |
| 熔点 | ℃ | -152.15 |
| 沸点 | ℃ | -82.2 |
| 液体密度(25℃) | 克/厘米3 | 0.669 |
| 蒸气压 (25℃) | MPa | 4.732 |
| 临界温度 | ℃ | 25.95 |
| 临界压力 | MPa | 4.832 |
| 临界密度 | 克/厘米3 | 1.221 |
| 沸点时蒸发潜热 | 千焦耳/千克 | 0.524 |
| 在水中溶解度 (25℃) | W% | 240 |
| 比热 (液体25℃) | 千焦耳/千克度 | 0.10 |
| ODP |  | 1.55 |
| GWP |  | 0 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |

R32

R32 is a colorless gas under ordinary temperature and a colorless & transparent liquid under the pressure of itself. It is mainly used as refrigerant which is the vital part to substitute R22. R32 is also an important component of blend refrigerant R407C and R410a. .

Properties：

| Index | Unit | R32 |
| --- | --- | --- |
| Chemical formula |  | CH2F2 |
| Molecular weight | g/mol | 52.02 |
| Freezing point | ℃ | -136 |
| Boiling point | ℃ | -51.7 |
| Specific gravity (25℃ liquid) | g/cm3 | 0.958 |
| Vapor pressure (25℃) | MPa | 1.689 |
| Critical temperature | ℃ | 78.10 |
| Critical pressure | MPa | 5.808 |
| Critical density | g/cm3 | 0.424 |
| Water solubility (25℃) | W% | 0.44 |
| ODP |  | 0 |
| GWP |  | 0.11 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| Odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides (Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing： 670KG/800L,780KG/926Lsteel cylinders, or ISO TANK.

PS：HS Code 29034200.00

性状用途：

在常温下为无色气体，在自身压力下为无色透明液体，用作制冷剂，是替代R22的重要组分，是混合制冷剂R407C、R410a的主要组成部分之一。

物理性质 ：

|  | 单位 | HFC-32 |
| --- | --- | --- |
| 分子式 |  | CH2F2 |
| 分子量 | g/mol | 52.02 |
| 冰点 | ℃ | -136 |
| 沸点 | ℃ | -51.7 |
| 液体密度(25℃) | 克/厘米3 | 0.958 |
| 蒸气压 (25℃) | MPa | 1.689 |
| 临界温度 | ℃ | 78.10 |
| 临界压力 | MPa | 5.808 |
| 临界密度 | 克/厘米3 | 0.424 |
| 在水中溶解度 (25℃) | W% | 0.44 |
| ODP |  | 0 |
| GWP |  | 0.11 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R123

R123 is a colorless gas under ordinary temperature and a colorless & transparent liquid under the pressure of itself, used as refrigerant, the vital part of blend working substances to substitute R11 and R113.

Properties：

| Index | Unit | R123 |
| --- | --- | --- |
| Chemical formula |  | CF3CH Cl2 |
| Molecular weight | g/mol | 152.91 |
| Melting point | ℃ |  |
| Boiling point | ℃ | 27.6 |
| Specific gravity (25℃) | g/cm3 | 1.458 |
| Vapor pressure (25℃) | MPa |  |
| Critical temperature | ℃ | 184 |
| Critical pressure | MPa | 3.605 |
| Critical density | g/cm3 | 0.549 |
| Vapor heat at boiling point | KJ/kg | 167.9 |
| Water solubility (25℃) | W% | 0.39 |
| Specific heat (25℃liquid) | KJ/kg.k | 0.985 |
| ODP |  | 0.02 |
| GWP |  | 0.02 |

Quality Specification：

| Index | Quality performance |
| --- | --- |
| Appearance | Colorless , not turbid |
| Odor | No strange stench |
| Purity % ≤ | 99.8 |
| Moisture % ≤ | 0.002 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |

Packing：125KG/100L, 250KG/200L steel cylinders

性状用途：

R123的物理性质与R11相似，是R11的替代品过渡。

物理性质 ：

|  | 单位 | HCFC-123 |
| --- | --- | --- |
| 分子式 |  | CF3CH Cl2 |
| 分子量 | g/mol | 152.91 |
| 熔点 | ℃ |  |
| 沸点 | ℃ | 27.6 |
| 液体密度(25℃) | 克/厘米3 | 1.458 |
| 蒸气压 (25℃) | KPa |  |
| 临界温度 | ℃ | 184 |
| 临界压力 | MPa | 3605 |
| 临界密度 | 克/厘米3 | 0.549 |
| 沸点时蒸发潜热 | 千焦耳/千克 | 167.9 |
| 在水中溶解度 (25℃) | W% | 0.39 |
| 比热 (液体25℃) | 千焦耳/千克度 | 0.985 |
| ODP |  | 0.02 |
| GWP |  | 0.02 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.002 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |

R125

R125 is a colorless gas under ordinary temperature and a colorless & transparent liquid under the pressure of itself, used as refrigerants, the vital part of mixed working substances to substitute R502&R22, and as flame-retardants to substitute Halon-1211 & Halon-1301.

Properties：

| Index | Unit | R125 |
| --- | --- | --- |
| Chemical formula |  | CHF2CF3 |
| Molecular weight | g/mol | 120.02 |
| Melting point | ℃ | -102.95 |
| Boiling point | ℃ | -48.45 |
| Specific gravity (25℃) | g/cm3 | 1.245 |
| Vapor pressure (25℃) | MPa | 1.371 |
| Critical temperature | ℃ | 66.05 |
| Critical pressure | MPa | 3.592 |
| Critical density | g/cm3 | 0.571 |
| Vapor heat at boiling point | KJ/kg | 165 |
| Water solubility (25℃) | W% | 0.09 |
| Specific heat (25℃liquid) | KJ/kg.kwh | 1.26 |
| ODP |  | 0 |
| GWP |  | 0.84 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：760KG/800L,870KG/926L steel cylinders or ISO TANK.

PS：HS Code 29034599.00, CAS 354-33-6.

性状用途：

在常温下为无色气体，在自身压力下为无色透明液体。HFC-125可作制冷剂，是混合工质的重要组份，用于替代CFC-502和HCFC-22；作灭火剂用于替代Halon-1211和Halon-1301。

物理性质 ：

|  | 单位 | HFC-125 |
| --- | --- | --- |
| 分子式 |  | CHF2 CF3 |
| 分子量 | g/mol | 120.02 |
| 熔点 | ℃ | -102.95 |
| 沸点 | ℃ | -48.45 |
| 液体密度(25℃) | 克/厘米3 | 1.245 |
| 蒸气压 (25℃) | MPa | 1.371 |
| 临界温度 | ℃ | 66.05 |
| 临界压力 | MPa | 3.592 |
| 临界密度 | 克/厘米3 | 0.571 |
| 沸点时蒸发潜热 | 千焦耳/千克 | 165 |
| 在水中溶解度 (25℃) | W% | 0.09 |
| 比热 (液体25℃) | 千焦耳/千克度 | 1.26 |
| ODP |  | 0 |
| GWP |  | 0.84 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R134a

R134a is internationally recognized as one of the main refrigerant working substances to substitute CFC-12. It does not contain chlorine atom, gives no bad effects against ozonosphere and has excellent safety performances, such as non-toxicity, non-inflammability, non-explosive ability , non-pungency, and non-corrosive ability. It is a colorless gas under ordinary temperature and a colorless & transparent liquid under the pressure of itself. As the substitute for CFC-12, it is often used in the refrigerating systems of auto air conditioners, commercial and industrial purposes, and as the vesicant, it is used to produce the heat-protective material made of rigid plastics, and also to prepare other blend refrigerants, such as R404A & R407C.

Properties：

| Index | Unit | (R134a) |
| --- | --- | --- |
| Chemical formula |  | CH2FCF3 |
| Molecular weight | g/mol | 102.0 |
| Boiling point 101.3KPa (℃) | ℃ | -26.2 |
| Freezing point 101.3KPa (℃) | ℃ | -96.6 |
| Critical temperature | ℃ | 101.1 |
| Critical pressure | KPa | 4066.6 |
| Saturated liquid density (25℃) | Kg/m3 | 1188.1 |
| Specific heat (25℃liquid) | KJ/kg.k | 1.42 |
| Critical density | g/cm3 | 0.512 |
| Vaporization heat at boiling point | KJ/kg | 215.0 |
| Water solubility (25℃) | W% | 0.15 |
| ODP |  | 0 |
| GWP |  | 0.29 |

Quality Specification：

| Index | Excellent Grade | Grade I |
| --- | --- | --- |
| Appearance | Colorless &transparent liquid |  |
| Purity % ≥ | 99.9 | 99.5 |
| Moisture % ≤ | 0.001 | 0.005 |
| Acidity (as HCl) % ≤ | 0.0001 | 0.0001 |
| Evaporation residue % ≤ | 0.01 | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |  |
| No condensable gas (V/V) % ≤ | 1.0 |  |

Packing：13.6kg, 800kg/L, 950kg/926L steel cylinders.

PS：HS Code 29033090.90.

性状用途：

HFC-134a是目前国际公认的CFC-12 的主要制冷工质之一，HFC-134a不含氯原子，对大气臭氧层不起破坏作用；具有良好的安全性能（不易燃，不爆炸，无毒，无刺激性，无腐蚀性）；在常温下为无色气体，在自身压力下为无色透明液体，是R12 的替代品。常用于汽车空调，商业和工业用制冷系统，以及作为发泡剂用于硬塑料保温材料生产，也可以用来配置其他混合制冷剂，如R404a和R407c等。

物理性质 ：

|  | 单位 | HFC-134a |
| --- | --- | --- |
| 分子式 |  | CH2 FCF3 |
| 分子量 | g/mol | 102.0 |
| 沸点 101.3kpa (℃) | ℃ | -26.2 |
| 冰点 101.3kpa (℃) | ℃ | -96.6 |
| 临界温度 | ℃ | 101.1 |
| 临界压力 | KPa | 4066.6 |
| 饱和液体密度(25℃) | Kg/m3 | 1188.1 |
| 比热 (液体25℃) | 千焦耳/千克度 | 1.42 |
| 临界密度 | 克/厘米3 | 0.512 |
| 沸点时蒸发潜热 | 千焦耳/千克 | 215.0 |
| 在水中溶解度 (25℃) | W% | 0.15 |
| ODP |  | 0 |
| GWP |  | 0.29 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 透明，无色液化气体 |
| 纯度 ( % ≥) | 99.9 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.0001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.0 |

R152a

R152a is a colorless gas under ordinary temperature, and a colorless & transparent liquid under the pressure of itself, easily solube in oil but poorly in water. It is mainly used as the  important raw material for refrigerants, aerosol propellants and synthesis of R142b. As the substitutive refrigerant for CFC-12, it will be a long-term one and have absolutely no bad effects against ozonosphere.

Properties：

| Index | Unit | R152a |
| --- | --- | --- |
| Chemical formula |  | CH3CHF2 |
| Molecular weight | g/mol | 66.1 |
| Melting point | ℃ | -24.23 |
| Liquid density (25℃) | g/cm3 | 0.90 |
| Vapor pressure (25℃) | MPa | 0.596 |
| Critical temperature | ℃ | 113.5 |
| Critical pressure | MPa | 4.52 |
| Critical density | g/cm3 | 0.368 |
| Vaporization heat at boiling point | KJ/kg | 324.2 |
| Water’s solubility (25℃) | W% | 0.3 |
| ODP |  | 0 |
| GWP |  | 0.3 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| Odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.0001 |
| Evaporation residue % ≤ | 0.01 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：320KG/400L&640KG/800L steel cylinders (≥2.0MPa), ISO TANK.

PS：UN NO.1030, HS Code 29034599.00, CAS 75-37-6, IMDG Page 2132, Class 2.1.

性状用途：

1，1-二氟乙烷在常温下为气体，在自身压力下为无色透明液体，易溶于油，难溶于水。高纯级R152a用作感温工质，优级R152a可用作制冷剂和气雾剂，也是合成R142b的重要原料。该产品可替代CFC-12作制冷剂，是对臭氧层完全无破坏的长久性替代品。

物理性质 ：

|  | 单位 | R152a |
| --- | --- | --- |
| 分子式 |  | CH3-CHF2 |
| 分子量 | g/mol | 66.1 |
| 沸点 | ℃ | -24.23 |
| 液体密度(25℃) | 克/厘米3 | 0.90 |
| 蒸气压 (25℃) | MPa | 0.596 |
| 临界温度 | ℃ | 113.5 |
| 临界压力 | MPa | 4.52 |
| 临界密度 | g/cm3 | 0.368 |
| 沸点时蒸发潜热 | KJ/kg | 324.2 |
| 在水中溶解度 (25℃) | %(m/m) | 0.3 |
| ODP |  | 0 |
| GWP |  | 0.03 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.95 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.0001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R406A

R406A is a colorless gas under ordinary temperature, and a colorless & transparent liquid under the pressure of itself,      mixed by R22, R142b and R600a. And it is the substitute for R502 and R12.

Properties：

|  | Unit | R406A |
| --- | --- | --- |
| Chemical formula |  | R-22/R-142b/R-600a |
| Molecular weight | g/mol | 89.86 |
| Melting point | ℃ | -32.7 |
| Critical temperature | ℃ | 116.5 |
| Critical pressure | MPa | 4.88 |
| Saturated liquid density (25℃) | g/cm3 |  |
| Critical density | g/cm3 | -0.456 |
| ODP |  | 0.036 |
| GWP |  | 0.33 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：10.7kg/30p steel cylinders.

It must be kept in cool, dry and well-ventilated places, away from sunshine and rainfall.

性状用途：

R406A混合制冷剂由R22，R142b和R600a混合而成，在常温下为无色气体，在自身压力下为无色透明液体，是R500，R12的替代品。

物理性质 ：

|  | 单位 | R406A |
| --- | --- | --- |
| 分子式 |  | R-22/R-142b/R-600a |
| 分子量 | g/mol | 89.86 |
| 沸点 | ℃ | -32.7 |
| 临界温度 | ℃ | 116.5 |
| 临界压力 | MPa | 4.88 |
| 饱和液体密度 (25℃) | g/cm3 |  |
| 临界密度 | g/cm3 | -0.456 |
| ODP |  | 0.036 |
| GWP |  | 0.33(HGWP) |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、澄清 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R404A

R404A is a colorless gas under ordinary temperature, and a colorless & transparent liquid under the pressure of itself,     mixed by R125, R134a and R143. It is a long-term substitute for R502, mainly used in the refrigerating systems of low & moderate temperature.

Properties：

|  | Unit | R404A |
| --- | --- | --- |
| Chemical formula |  | R-134a/R-125/R-32 |
| Molecular weight | g/mol |  |
| Melting point | ℃ | -46.1 |
| Critical temperature | ℃ | 72.4 |
| Critical pressure | MPa | 3.69 |
| Saturated liquid density (25℃) | g/cm3 | 1.045 |
| ODP |  | 0 |
| GWP |  | 0.35 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：10.9kg/30p steel cylinders or ISO TANK.

It must be kept in cool, dry and well-ventilated places, away from sunshine and rainfall.

性状用途：

R404A由R125，R134a和R143混合而成，在常温下为无色气体，在自身压力下为无色透明液体，是R502的长期替代品，主要用于低、中温制冷系统。

物理性质 ：

|  | 单位 | R404A |
| --- | --- | --- |
| 分子式 |  | R-134a/R-125/R-32 |
| 分子量 | g/mol |  |
| 沸点 | ℃ | -46.1 |
| 临界温度 | ℃ | 72.4 |
| 临界压力 | MPa | 3.69 |
| 饱和液体密度 (25℃) | g/cm3 | 1.045 |
| ODP |  | 0 |
| GWP |  | 0.35(HGWP) |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、澄清 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.0001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R410A

R410a is a clorless gas under ordinary temperature, and a colorlss & Transparent Liquid under the pressure of itslf, mixed by HFC-32 and HFC-125. And it is the substitute for R22, mainly used in the air conditioners and other refrigeratig systems.

Properties：

| Index | Unit | R410A |
| --- | --- | --- |
| Chemical formula |  | R-32/R-125 |
| Molecular weight | g/mol | 72.58 |
| Boiling point | ℃ | -51.53 |
| Critical temperature | ℃ | 72.13 |
| Critical pressure | MPa | 4.93 |
| Saturated liquid density (25℃) | g/cm3 | 1.062 |
| Liquid specific heat (25℃) | KJ/(Kg ·℃) | 1.84 |
| Equip pressure specific heat (Cp) (25℃) 101.3kPa | KJ/(Kg ·℃) | 0.832 |
| Critical density | g/cm3 | 0.489 |
| Vaporization heat at boiling point | KJ/Kg | 276.2 |
| ODP |  | 0 |
| GWP |  | 0.42 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| Odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：11.3kg/30P unrefillable steel cylinders;760kg/800L,870kg/926L returnable steel

cylinders, or ISO TANK. The filling index is less than0.95kg/L. It must be kept in cool, dry and well-ventilated places, away      from sunshine and rainfall.

性状用途：

R410A混合制冷剂是由R32和R125混合而成，在常温下为无色气体，在自身压力下为无色透明液体，是R22的替代品，主要用于空调和制冷系统。

物理性质 ：

|  | 单位 | R410A |
| --- | --- | --- |
| 分子式 |  | R-32/ R-125 |
| 分子量 | g/mol | 72.58 |
| 沸点 | ℃ | -51.53 |
| 临界温度 | ℃ | 72.13 |
| 临界压力 | MPa | 4.93 |
| 饱和液体密度 (25℃) | g/cm3 | 1.062 |
| 液体比热 (25℃) | KJ/(Kg·℃) | 1.84 |
| 等压蒸气比热 (25℃) | KJ/(Kg·℃) | 0.832 |
| 临界密度 | g/cm3 | 0.489 |
| 沸点时蒸发潜热 | KJ/kg | 276.2 |
| ODP |  | 0 |
| GWP |  | 0.42 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、澄清 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R407C

R407C is a colorless gas under ordinary temperature, and a colorless & transparent liquid under the pressure of itself,        mixed by R32, R134aand R125. R407Cis the long-term substitute for R22, mainly used in the air conditioner and other non-centrifugal refrigerating systems

Properties：

|  | Unit | R407C |
| --- | --- | --- |
| Chemical formula |  | R-32/R-134a/R-125 |
| Molecular weight | g/mol | 86.2 |
| Melting point | ℃ | -43.4 |
| Critical temperature | ℃ | 86.2 |
| Critical pressure | MPa | 4.62 |
| Saturated liquid density (30℃) | g/cm3 | 1.139 |
| Critical density | g/cm3 | 0.527 |
| ODP |  | 0 |
| GWP |  | 1600 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：10.9kg/30p, 950kg/1000L, 870kg/926L steel cylinders or ISO TANK.

It must be kept in cool, dry and well-ventilated places, away from sunshine and rainfall.

性状用途：

R407C由R32，R134a和R125混合而成，在常温下为无色气体，在自身压力下为无色透明液体，是R22的长期替代品，主要用于空调、非离心式制冷系统。

物理性质 ：

|  | 单位 | R407C |
| --- | --- | --- |
| 分子式 |  | R-32/R-134a/R-125 |
| 分子量 | g/mol | 86.2 |
| 沸点 | ℃ | -43.4 |
| 临界温度 | ℃ | 86.2 |
| 临界压力 | MPa | 4.62 |
| 饱和液体密度 (30℃) | g/cm3 | 1.139 |
| 临界密度 | g/cm3 | 0.527 |
| ODP |  | 0 |
| GWP |  | 0.36 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、澄清 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R507

R507 is a colorless gas under ordinary temperature, and a colorless & transparent liquid under the pressure of itself,        mixed by R143 and R125. It is the substitute for R502, mainly used in the low temperature refrigerating systems.

Properties：

|  | Unit | R507 |
| --- | --- | --- |
| Chemical formula |  | R-143/R-125 |
| Molecular weight | g/mol | 98.86 |
| Melting point | ℃ | -46.7 |
| Critical temperature | ℃ | 70.9 |
| Critical pressure | MPa | 3.79 |
| Saturated liquid density (30℃) | g/cm3 | 1.06 |
| Critical density | g/cm3 | 0.5 |
| ODP |  | 0 |
| GWP |  | 0.96 |

Quality Specification：

| Index | Excellent Grade |
| --- | --- |
| Appearance | Colorless , not turbid |
| odor | No strange stench |
| Purity % ≥ | 99.8 |
| Moisture % ≤ | 0.001 |
| Acidity (as HCl) % ≤ | 0.00001 |
| Evaporation residue % ≤ | 0.01 |
| Chlorides(Cl-) test % ≤ | 0.0003 |
| No condensable gas (V/V) % ≤ | 1.5 |

Packing：11.3kg/30p steel cylinders or ISO TANK.

It must be kept in cool, dry and well-ventilated places, away from sunshine and rainfall.

性状用途：

R507混合制冷剂是由R143和R125混合而成，在常温下为无色气体，在自身压力下为无色透明液体，是R502的替代品，主要用于低温制冷系统。

物理性质 ：

|  | 单位 | R507 |
| --- | --- | --- |
| 分子式 |  | R-143/R-125 |
| 分子量 | g/mol | 98.86 |
| 沸点 | ℃ | -46.7 |
| 临界温度 | ℃ | 70.9 |
| 临界压力 | MPa | 3.79 |
| 饱和液体密度 (30℃) | g/cm3 | 1.06 |
| 临界密度 | g/cm3 | 0.5 |
| ODP |  | 0 |
| GWP |  | 0.96 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、澄清 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.8 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |
| 氯化物(Cl-) ( % ≤) | 0.0003 |
| 不凝性气体的体积分数 ( % ≤) | 1.5 |

R600a

R600a is a HFC and CFC free refrigerant, providing the most environmentally friendly refrigerant available. It is extremely    energy efficient, and has high cooling ability, low electricity waste.

Properties：

| Index | Unit | R600a |
| --- | --- | --- |
| Chemical formula |  | C4H10 |
| Molecular weight | g/mol | 58.12 |
| Critical temperature | ℃ | 134.98 |
| Critical pressure | KPa | 3660 |
| Saturated liquid density (25℃) | Kg/m3 | 551 |
| Specific heat (25℃liquid) | KJ/kg·k | 2.38 |
| Critical density | g/cm3 | 0.221 |
| Vaporization heat at boiling point | KJ/kg | 366.5 |
| ODP |  | 0 |
| GWP |  | 0.1 |

Quality Specification：

| Index | Grade Ⅰ |
| --- | --- |
| Appearance | Colorless & not turbid |
| odor | No strange stench |
| Purity % ≥ | 99.5 |
| Moisture ppm % ≤ | 0.001 |
| Acidity (as HCl) ppm % ≤ | 0.0001 |
| Evaporation residue ppm % ≤ | 0.01 |

Packing：20kg/40L, 460kg/926L, returnable steel cylinders.

性状用途：

R600a（高纯度异丁烷）是一种性能优异的制冷剂，其特点是冷却能力强，耗电量低，负载温度回升速度慢。

物理性质 ：

|  | 单位 | R600a |
| --- | --- | --- |
| 分子式 |  | C4H10 |
| 分子量 | g/mol | 58.12 |
| 沸点 101.3kpa (℃) | ℃ | -11.80 |
| 临界温度 | ℃ | 134.98 |
| 临界压力 | KPa | 3660 |
| 饱和液体密度 (25℃) | Kg/m3 | 551 |
| 比热 (25℃液体) | KJ/Kg· K | 2.38 |
| 临界密度 | g/cm3 | 0.221 |
| 沸点时蒸发潜热 | KJ/kg | 366.5 |
| ODP |  | 0 |
| GWP |  | 0.1 |

质量指标 ：

| 指标名称 | 优等品 |
| --- | --- |
| 外观 | 无色、不浑浊 |
| 气味 | 无异臭 |
| 纯度 ( % ≥) | 99.5 |
| 水份( % ≤) | 0.001 |
| 酸度 ( 以HCl计 ) ,( % ≤) | 0.00001 |
| 蒸发残留物 ( % ≤) | 0.01 |